# RemasLogger Breakpoint Audit

This audit maps source-history breakpoints to engineer-facing MTOS manual ranges. Commit hashes are included for traceability only; engineers should choose manuals by MTOS version using README.md.

## Confirmed Manual Breakpoints

| Breakpoint | First confirmed MTOS tag | Source evidence | Impact |
| --- | --- | --- | --- |
| Legacy Remas output before DP_STRUCT UDP | Before v1.5.3.4 | f048636b3 introduced serial Remas support in 2008; older send paths used serial / legacy Remas network concepts. | Historical manual only. Do not assume DP_STRUCT UDP for these versions. |
| UDP moved to DP_STRUCT | v1.5.3.4 | 12c3ebd61 changes the Remas UDP send call to Send(DP_STRUCT, NOTADD_SEND_IDENT, ...). | First confirmed DP_STRUCT UDP manual range. Payload is raw packed remas_struct bytes without struct_ident. |
| Extended power/battery/follow-target layout | v1.5.25 | Tagged v1.5.25 struct contains bBatteryReady, dStateOfCharge, dRemainingEnergy, dRemainingTimeMin, dRemainingTimeMinWorstCase, and dGeneratorFuel. | New payload tail fields; split from v1.5.3.4 range. |
| Speed sensor and Voith layout | v1.6.0.0/MTOS | Tagged source contains lSpeedSensor[2][MAX_NO_SPEED] and Voith pitch command/feedback arrays before timestamp/checksum. | New payload tail fields; split from v1.5.25 range. |
| Current active layout | v1.6.18 | Tagged source contains current active tail fields including dExtCtrlSpdRef, dExtCtrlAccRef, bAutonomyMode, trackingControllerData, and dRollStabilisationCommand. | Latest manual range starts here. |

## Audited Non-Breakpoints

| Source history | Finding | Manual decision |
| --- | --- | --- |
| f3678ef05 / 6699d8ead 10 Hz Remas branch history | The commits are visible on archived/feature branches but were not found as first-class MTOS release breakpoints in the current master release path. | Document frequency behavior where present, but do not split a release manual solely on this branch-only history. |
| 2b0f1b6a3 checksum semantic change | The commit was not reachable from current master or public release tags during this audit. | Keep dwCRC documented as checksum/raw in manuals; verify branch-specific vessels separately. |
| 2024-2025 playback/logging changes (#4907, #5435) | These touched playback_struct, logging setup, or binary logging paths. The active RemasLogger UDP send path still sends remas_struct. | Do not create separate RemasLogger UDP manuals unless the active remas_struct layout or send path changed. |
| remas_struct2 and playback_struct definitions | Present in MTStructDef.h, but not sent by the active RemasLogger UDP path. | Include as related structs, not as primary manual selection rows. |

## Verification Pointers

Use these files for future audits:

Common/Definition/MTStructDef.h: packed struct definitions and field order.

OS/MTOSView/MTOSDoc.cpp: Remas snapshot population and send path.

OS/MTOSView/MTFunction.cpp and function.ini: RemasLogger setting semantics.

Configuration.base.DP/Network/communication.ini: DP_STRUCT network channel configuration.

Only add a new manual range when the active RemasLogger payload layout, transport/channel, frequency behavior, or field semantics change in a released MTOS version.