# RemasLogger Payload Manual - MTOS v1.6.0.0 to v1.6.17.x

Applies to MTOS versions: v1.6.0.0 through v1.6.17.x Use this manual when the vessel MTOS version is in this range. Payload: remas_struct Transport: DP_STRUCT UDP, or serial when RemasLogger = 1 Frequency: 1 Hz / 2 Hz / 10 Hz depending on RemasLogger value in the vessel release Superseded by: remas-logger-mtos-v1.6.18-and-newer.md

## Layout Summary

This range adds speed-sensor and Voith pitch arrays to the v1.5.25 layout. It is still older than the current v1.6.18+ layout.

Compared with v1.6.18 and newer, this range does not contain:

dGpsVruComp[2][MAX_NO_GPS]

dGpsXYDist[2][MAX_NO_GPS]

dExtCtrlSpdRef, dExtCtrlAccRef, bAutonomyMode

trackingControllerData

dRollStabilisationCommand

## Fields Added In This Range

These fields are present after dGeneratorFuel[MAX_NO_GENERATOR] and before i64TimeStamp:

| Field | C type | Format / unit |
| --- | --- | --- |
| lSpeedSensor[2][MAX_NO_SPEED] | float[2][3] | m/s, [0]=along, [1]=athwart |
| dVoithLongitudalPitchCommand[MAX_NO_THRUSTER] | float[10] | -1..1 |
| dVoithTransversePitchCommand[MAX_NO_THRUSTER] | float[10] | -1..1 |
| dVoithLongitudalPitchFeedback[MAX_NO_THRUSTER] | float[10] | -1..1 |
| dVoithTransversePitchFeedback[MAX_NO_THRUSTER] | float[10] | -1..1 |

## Field Order

The first fields match remas-logger-mtos-v1.5.25-to-v1.5.29.md through dGeneratorFuel[MAX_NO_GENERATOR]. The complete tail for this range is:

bool bAllocCombinatorMode
bool bEngineInVariableMode[MAX_NO_MAIN_ENGINE]
bool bBatteryReady[MAX_NO_GENERATOR]
float dStateOfCharge[MAX_NO_GENERATOR]
float dRemainingEnergy[MAX_NO_GENERATOR]
float dRemainingTimeMin[MAX_NO_GENERATOR]
float dRemainingTimeMinWorstCase[MAX_NO_GENERATOR]
float dGeneratorFuel[MAX_NO_GENERATOR]
float lSpeedSensor[2][MAX_NO_SPEED]
float dVoithLongitudalPitchCommand[MAX_NO_THRUSTER]
float dVoithTransversePitchCommand[MAX_NO_THRUSTER]
float dVoithLongitudalPitchFeedback[MAX_NO_THRUSTER]
float dVoithTransversePitchFeedback[MAX_NO_THRUSTER]
SYSTEMTIME i64TimeStamp
short dwCRC
long dwBytePattern1
long dwBytePattern2

## Transport And Value Mappings

Use the transport rules and shared enum/value mappings in remas-logger-mtos-v1.6.18-and-newer.md, but apply only the field order documented here for binary decoding.