# RemasLogger Payload Manual - MTOS v1.5.3.4 to v1.5.24.x

Applies to MTOS versions: v1.5.3.4 through v1.5.24.x Use this manual when the vessel MTOS version is in this range. Payload: remas_struct Transport: DP_STRUCT UDP when RemasLogger = 2; serial when RemasLogger = 1 Frequency: 1 Hz in the audited tagged v1.5.3.4 send path Superseded by: remas-logger-mtos-v1.5.25-to-v1.5.29.md

## Transport

This is the first confirmed MTOS release range where RemasLogger UDP sends raw packed remas_struct bytes on the DP_STRUCT channel using NOTADD_SEND_IDENT.

For the usual OS1 UDP setup:

RemasLogger         = 2
RemasLoggerOSNumber = 1

The sender calls Send(DP_STRUCT, NOTADD_SEND_IDENT, (long*)&sendData, sizeof(remas_struct), 1). Consumers must not expect the normal struct_ident application header.

## Layout Differences From Current

Navigator arrays are fixed at 10 slots in the audited v1.5.3.4 source, not MAX_NO_PLAYBACK_NAV = 15.

Vessel, navigator, target, carrot, and setpoint positions are float, not double.

VRU data is dVRU[2][MAX_NO_VRU] for roll and pitch only; current layouts use three rows including heave.

This range does not contain the later DP/TCS in-use arrays, estimator arrays, follow-target fields, battery fields, speed-sensor fields, Voith fields, autonomy, track-controller data, or roll-stabilisation command.

## Field Order

| Field | C type | Format / unit |
| --- | --- | --- |
| lMainMode | short | Main mode value |
| lSubMode | short | Sub mode value |
| dPositionDeviation[2] | float[2] | m |
| dHeadingDeviation | float | deg |
| dVesselPos[2] | float[2] | deg |
| dHeading | float | deg |
| dSpeed[2] | float[2] | m/s |
| dRotSpeed | float | deg/min |
| NavigatorStatus[10] | short[10] | SensorStatus |
| bNavigatorNotAccepted[10] | bool[10] | boolean |
| lTargetID[10] | short[10] | config-dependent id |
| lTargetType[10] | short[10] | target/transponder type |
| dNavigatorPosition[2][10] | float[2][10] | deg |
| dTargetPosition[2][10] | float[2][10] | deg |
| dGyroHeading[MAX_NO_GYRO] | float[] | deg |
| lGyroStatus[MAX_NO_GYRO] | short[] | SensorStatus |
| bGyroNotAccepted[MAX_NO_GYRO] | bool[] | boolean |
| dVRU[2][MAX_NO_VRU] | float[2][] | roll/pitch deg |
| lVRUStatus[MAX_NO_VRU] | short[] | SensorStatus |
| dWind[2][MAX_NO_WIND] | float[2][] | m/s, deg |
| lWindStatus[MAX_NO_WIND] | short[] | SensorStatus |
| dWindDirection | float | deg |
| dWindSpeed | float | m/s |
| dCurrentDirection | float | deg |
| dCurrentSpeed | float | m/s |
| dCarrotPosition[2] | float[2] | deg |
| dCarrotHeading | float | deg |
| dPositionSetpoint[2] | float[2] | deg |
| dHeadingSetpoint | float | deg |
| lThrusterType[MAX_NO_THRUSTER] | short[] | ThrusterType |
| lThrusterStatus[MAX_NO_THRUSTER] | short[] | ThrStatus |
| dRPMCommand[MAX_NO_THRUSTER] | float[] | -1..1 |
| dRPMFeedback[MAX_NO_THRUSTER] | float[] | -1..1 |
| dPitchCommand[MAX_NO_THRUSTER] | float[] | -1..1 |
| dPitchFeedback[MAX_NO_THRUSTER] | float[] | -1..1 |
| dRelativeThrustCommand[MAX_NO_THRUSTER] | float[] | -100..100 % |
| dRelativeThrustFeedback[MAX_NO_THRUSTER] | float[] | -100..100 % |
| dAzimuthCommand[MAX_NO_THRUSTER] | float[] | deg |
| dAzimuthFeedback[MAX_NO_THRUSTER] | float[] | deg |
| dThrPowerFeedback[MAX_NO_THRUSTER] | float[] | kW |
| bAxisControl[3] | bool[3] | boolean |
| dForceCmd[3] | float[3] | t |
| ForceFdb[3] | float[3] | t |
| dDebugData[10] | float[10] | raw |
| sTCVoteStatus[6] | short[6] | vote bitmask |
| sCCPrefCC[3] | short[3] | preferred CC bitmask |
| sCCVoteBuffer[3][5] | short[3][5] | raw vote values |
| dCarrotVelocity[3] | float[3] | m/s, m/s, deg/min |
| dBallisticSpeed[3] | float[3] | m/s, m/s, deg/min |
| dControllerP[3] | float[3] | t |
| dControllerD[3] | float[3] | t |
| dControllerI[3] | float[3] | t |
| dControllerWindFF[3] | float[3] | t |
| dControllerAccFF[3] | float[3] | t |
| dGeneratorLoad[MAX_NO_GENERATOR] | float[] | kW |
| dDieselLoad[MAX_NO_MAIN_ENGINE] | float[] | kW |
| bGenBreakerStatus[MAX_NO_GENERATOR] | bool[] | boolean |
| bThrBreakerStatus[MAX_NO_THRUSTER] | bool[] | boolean |
| bBusBreakerStatus[MAX_NO_BUSES] | bool[] | boolean |
| lNoOfGenConnected[MAX_NO_BUSES] | short[] | count |
| lNoOfThrConnected[MAX_NO_BUSES] | short[] | count |
| lCCInUse | short | device id, usually CC1=26..CC3=28 |
| lOSInCmd | short | device id, usually OS1=1..OS25=25 |
| i64TimeStamp | SYSTEMTIME | Windows timestamp |
| dwCRC | short | checksum/raw |
| dwBytePattern1 | long | marker |
| dwBytePattern2 | long | marker |

## Shared Value Tables

Use remas-logger-mtos-v1.6.18-and-newer.md for enum/value mappings. For this range, ignore mappings for fields that do not exist in the table above.