# RemasLogger Payload Manual - MTOS v1.6.18 and newer

Applies to MTOS versions: v1.6.18 and newer, including current master / v1.7.1.8. Use this manual when the vessel MTOS version is v1.6.18 or newer. Payload: remas_struct Transport: DP_STRUCT UDP, or serial when RemasLogger = 1 Frequency: 1 Hz / 2 Hz / 10 Hz depending on RemasLogger value Superseded by: not superseded in the audited release history.

This document describes the binary payload sent when function.ini enables the Remas logger as UDP output from an OS, for example:

RemasLogger         = 2    # 1=Serial, 2=UDP 1 Hz, 3=UDP 2 Hz, 4=UDP 10 Hz
RemasLoggerOSNumber = 1    # Only this OS outputs Remas data

The active UDP output is one packed C/C++ structure: remas_struct.

Source references:

OS\MTOSView\MTOSDoc.cpp: CaptureRemasSnapshot() fills remas_struct.

OS\MTOSView\MTOSDoc.cpp: RemasLogger sends remas_struct on the DP_STRUCT network when RemasLogger >= 2.

Common\Definition\MTStructDef.h: remas_struct, remas_struct2, playback_struct, and nested structs.

Common\Definition\MTThrusterPackDefinition.h: thruster enums and limits.

Common\Definition\MTSensorPackDefinitions.h: sensor enums and limits.

Common\Definition\MTModeDefinitions.h: mode constants.

Common\Definition\MTUnitsDef.h: OS/CC/TCSCC/network device IDs.

Common\Definition\MTTCSDef.h: TCSCC/DPCC status struct.

## Source History

This manual is the latest confirmed active RemasLogger UDP layout. The release breakpoint is v1.6.18, where the active remas_struct contains the current tail fields dExtCtrlSpdRef, dExtCtrlAccRef, bAutonomyMode, trackingControllerData, and dRollStabilisationCommand before timestamp/checksum markers.

Audited breakpoints:

| MTOS version | Source change | Manual impact |
| --- | --- | --- |
| v1.5.3.4 | UDP Remas output changed to DP_STRUCT (12c3ebd61). | First confirmed DP_STRUCT UDP manual range. |
| v1.5.25 | Battery, remaining-time, and generator-fuel fields are present. | New payload tail fields before timestamp/checksum. |
| v1.6.0.0 | Speed sensor and Voith pitch arrays are present. | New payload tail fields before timestamp/checksum. |
| v1.6.18 | Current active layout with track-controller and roll-stabilisation fields is present. | Latest manual range starts here. |

Notes from Git history that did not create a separate released manual range:

RemasLogger = 3/4 frequency modes were found in branch history for 2 Hz / 10 Hz handling, but the audited public release tags already expose the current RemasLogger values in this latest range.

#2273 Checksum of Config files added to Remas struct changes dwCRC semantics in branch history, but the commit audited in this repository is not reachable from current master or a public release tag. Treat dwCRC as a raw/checksum field unless validating a vessel-specific branch.

2024-2025 playback/logging commits touched playback_struct and logging paths; only changes to the active remas_struct send path were used as manual breakpoints.

The local working tree currently contains unreleased remas_struct edits in Common\Definition\MTStructDef.h such as sDPClass and warning/alarm fields. Those fields are not in released HEAD / v1.7.1.8 source and are not assigned to an MTOS manual range here.

## Transport

RemasLogger = 2 sends the raw remas_struct payload over UDP at 1 Hz. RemasLogger = 3 sends at 2 Hz, and RemasLogger = 4 sends at 10 Hz. The selected source OS is controlled by RemasLoggerOSNumber.

The configured network section is DP_STRUCT. In the base DP configuration it is multicast 239.192.239.21, local PORT = 10000, REMOTE_PORT = 10001.

The send call uses NOTADD_SEND_IDENT, so the UDP payload is the packed struct bytes only, without the normal struct_ident application header.

## Structs Available

| Struct | Availability | Notes |
| --- | --- | --- |
| remas_struct | Active RemasLogger UDP payload | Sent by RemasLogger >= 2 on DP_STRUCT. Packed with #pragma pack(push,1). |
| track_controller_struct | Nested in remas_struct | Field trackingControllerData. |
| remas_struct2 | Defined, legacy/alternate layout | Uses wider long/double fields and is not sent by the current RemasLogger UDP path. |
| playback_struct | Defined for binary playback/logging | Similar to remas_struct, with additional playback/logging fields. Not the active RemasLogger UDP payload. |

## Important Constants

| Name | Value | Meaning |
| --- | --- | --- |
| MAX_NO_PLAYBACK_NAV | 15 | Navigator slots in Remas/playback data. |
| MAX_NO_GPS | 4 | GPS compensation/distance slots. |
| MAX_NO_GYRO | 5 | Gyro slots. |
| MAX_NO_WIND | 5 | Wind sensor slots. |
| MAX_NO_VRU | 5 | VRU slots. |
| MAX_NO_SPEED | 3 | Speed sensor slots. |
| MAX_NO_ROT | 3 | ROT sensor slots. |
| MAX_NO_EXT_FORCE | 2 | External force sensor slots. |
| MAX_NO_THRUSTER | 10 | Thruster slots. |
| MAX_NO_GENERATOR | 10 | Generator slots. |
| MAX_NO_MAIN_ENGINE | 4 | Main engine slots. |
| MAX_NO_BUSES | 8 | Bus slots. |

Unused sensor/thruster array entries are filled with status/type -1 and value sentinels such as FLT_MAX or DBL_MAX.

## Value Encoding Conventions

Fields with float, double, and count formats are continuous numeric values in the units shown in the field table. They do not have a finite list of possible values unless the table or source comment gives a range, for example -1..1, -100..100 %, or 0..1.

All bool fields are encoded as C++ bool: false = 0, true = 1.

Unused entries use these sentinels where populated by CaptureRemasSnapshot():

| Sentinel | Meaning |
| --- | --- |
| -1 | Unused or unavailable id/status/type slot. |
| FLT_MAX | Unused float slot. |
| DBL_MAX | Unused double slot. |

dwBytePattern1 and dwBytePattern2 are fixed markers:

| Field | Hex value | ASCII |
| --- | --- | --- |
| dwBytePattern1 | 0x4D544D54 | MTMT |
| dwBytePattern2 | 0x48415348 | HASH |

## Enum and Status Values

### ThrStatus

Defined in MTThrusterPackDefinition.h:

| Name | Value |
| --- | --- |
| UNAVAIL | 0 |
| RUNNING | 1 |
| READY | 2 |
| IN_USE | 3 |
| GEARREADY | 4 |

### ThrusterType

Defined in MTThrusterPackDefinition.h:

| Name | Value |
| --- | --- |
| TUNNEL | 1 |
| AZIMUTH | 2 |
| MAINPROP | 3 |
| RUDDER | 4 |
| COMBI | 5 |
| VOITH | 6 |

### SensorStatus

Defined in MTSensorPackDefinitions.h:

| Name | Value |
| --- | --- |
| SENS_UNAVAIL | 0 |
| SENS_READY | 1 |
| SENS_CALIB | 2 |
| SENS_ENABLED | 3 |
| SENS_IN_USE | 4 |

### Target / Transponder Types

Defined in MTSensorPackDefinitions.h. Used by lTargetType.

| Name | Value |
| --- | --- |
| ADMINISTRATOR_TP | 0 |
| SSBL_TP | 1 |
| LBL_TP | 2 |
| ROV_TP | 3 |
| VESSEL_TP | 4 |
| NAVIGATOR_TP | 5 |
| FOLLOW_TP | 6 |
| INS_TP | 7 |

### Main Modes

Defined in MTModeDefinitions.h:

| Name | Value |
| --- | --- |
| MANUAL_LEVERS | 1 |
| STANDBY | 2 |
| JOYSTICK | 4 |
| DP | 8 |
| TRANSIT | 16 |

### Sub Modes

Defined in MTModeDefinitions.h:

| Name | Value |
| --- | --- |
| NO_SUB_MODE | 0 |
| RELAXED | 1 |
| FOLLOW_TARGET | 2 |
| FOLLOW_LINE | 4 |
| SPEED_FROM_LEVERS | 8 |
| SPEED_FROM_JOYSTICK | 16 |
| AUTO_SPEED | 32 |
| DP_TRACK | 64 |
| SEISMIC_TRACK | 128 |
| CABLE_TRACK | 256 |
| AUTOCROSS | 512 |
| AUTOSAIL | 1024 |

### DP Class

Defined in MTModeDefinitions.h. This mapping is included for branches that expose DP class in Remas/playback data; released v1.6.18+ RemasLogger UDP layouts audited here do not include sDPClass in remas_struct.

| Name | Value |
| --- | --- |
| DP_CLASS_OFF | 0 |
| DP_CLASS_2 | 1 |
| DP_CLASS_3 | 2 |

### Follow Target Mode

Defined in MTModeDefinitions.h. Used by lFTMode.

| Name | Value |
| --- | --- |
| FOLLOW_PLATFORM | 0 |
| FOLLOW_ROV | 1 |

### Device IDs

Defined in MTUnitsDef.h. These are the values used by lOSInCmd, lCCInUse, lTCSCCInUse, and the TCSCC/DPCC in-use fields.

| Group | Values |
| --- | --- |
| OS units | OS1=1 through OS25=25. |
| DP CC units | CC1=26, CC2=27, CC3=28. |
| Thruster cards | TC1=37 through TC29=65. |
| DP struct network id | DP_STRUCT=90. |
| Remas network id | REMAS=94. |
| TCSCC units | TCSCC1=100, TCSCC2=101, TCSCC3=102. |

For example, lCCInUse = 26 means CC1 is in use.

### Vote Status

Defined in MTSensorPackDefinitions.h.

| Name | Value |
| --- | --- |
| NOT_CONFIGURED | 0 |
| VOTE_OK | 1 |
| VOTE_ERROR | 2 |
| VOTE_TIME_OUT | 3 |

### Preferred CC Bitmask

Used by sCCPrefCC / thr_voting_struct.sPrimID. It is a preference bitmask, not a device id.

| Value | Meaning |
| --- | --- |
| 0 | No explicit preference. |
| 1 | Prefer first CC in the voting group: CC1 or TCSCC1, depending on context. |
| 2 | Prefer second CC in the voting group: CC2 or TCSCC2, depending on context. |
| 4 | Prefer third CC in the voting group: CC3 or TCSCC3, depending on context. Also forced for CC3 in fire mode in DPCC voting. |

### Vote Result Bitmask

Used by sTCVoteStatus / thr_voting_reply_struct.sVoteRes.

| Bit/value | Meaning |
| --- | --- |
| 1 | First CC in group is in use (CC1 for DPCC voting, TCSCC1 for TCSCC voting). |
| 2 | Second CC in group is in use (CC2 or TCSCC2). |
| 4 | Third CC in group is in use (CC3 or TCSCC3). |
| 16 | First CC in group has vote error / voted out. |
| 32 | Second CC in group has vote error / voted out. |
| 64 | Third CC in group has vote error / voted out. |
| 256 | First CC in group timed out. |
| 512 | Second CC in group timed out. |
| 1024 | Third CC in group timed out. |

Values can be combined. For example, 273 = 1 + 16 + 256 means first CC is selected, has error, and has timed out.

### Vote Buffer Values

Used by sCCVoteBuffer[3][5] / thr_voting_struct.sVoteData.

| Index | Meaning |
| --- | --- |
| 0 | Surge command vote value. |
| 1 | Sway command vote value. |
| 2 | Yaw command vote value. |
| 3 | Thruster/status vote value. |
| 4 | Spare / sensor-count vote value, depending on voting source. |

DPCC voting buffers are generated as signed byte-sized ramp values and clamped to -63..63 before being stored as short. Some TCSCC paths pass thruster voting ramp buffers as short; treat these as raw signed vote/ramp values unless the sender-specific TCSCC implementation is also decoded.

### TCSCC/DPCC Status Values

Used by sDPCC_TCSCCInUse and sTCSCC_DPCCInUse. These fields copy tcsCC_dpCC_status_struct.lStatus[0]:

| Field | Source value | Expected values |
| --- | --- | --- |
| sDPCC_TCSCCInUse[i] | DPCC value received from TCSCC status messages | DP CC device id: CC1=26, CC2=27, or CC3=28; 0 before data is received. |
| sTCSCC_DPCCInUse[i] | TCSCC value received from DPCC status messages | TCSCC device id: TCSCC1=100, TCSCC2=101, or TCSCC3=102; 0 before data is received. |

### Config-Dependent IDs

These fields are runtime/configuration IDs and do not have a fixed global enum:

| Field | Value source |
| --- | --- |
| lTargetID[] | Local navigator target/transponder id. |
| lFTTarget1, lFTTarget2, lFTTransponderID | Follow-target target/transponder ids. |
| lFTAdmin1, lFTAdmin2, lFTHPRAdminID | Navigator/admin/HPR ids. |
| lOpPointNumber | Operator point number configured in the DP system. |

## remas_struct

remas_struct is defined with #pragma pack(push,1). Consumers must use the same packing and the same C type sizes as the sender.

| Field | C type | Format / unit | Notes |
| --- | --- | --- | --- |
| lMainMode | short | enum/bit value | See Main Modes. |
| lSubMode | short | enum/bit value | See Sub Modes. |
| dPositionDeviation[2] | float[2] | m | Surge and sway position deviation. In transit track mode, [0]=DTW, [1]=XTE. |
| dHeadingDeviation | float | deg | Heading deviation. In transit track mode, set from CTS. |
| dVesselPos[2] | double[2] | deg | [0]=latitude, [1]=longitude. |
| dHeading | float | deg | Vessel heading. |
| dSpeed[2] | float[2] | m/s | [0]=surge/along, [1]=sway/athwart. |
| dRotSpeed | float | deg/min | Vessel rate of turn. |
| NavigatorStatus[MAX_NO_PLAYBACK_NAV] | short[15] | SensorStatus | Navigator/reference-system status. See SensorStatus. Unused slots are -1. |
| bNavigatorNotAccepted[MAX_NO_PLAYBACK_NAV] | bool[15] | boolean | Navigator not-accepted flags. |
| lTargetID[MAX_NO_PLAYBACK_NAV] | short[15] | id | Target/transponder id for local navigators. See Config-Dependent IDs. Unused slots are -1. |
| lTargetType[MAX_NO_PLAYBACK_NAV] | short[15] | enum/id | Target/transponder type. See Target / Transponder Types. Unused slots are -1. |
| dNavigatorPosition[2][MAX_NO_PLAYBACK_NAV] | double[2][15] | deg | [0][i]=latitude, [1][i]=longitude. Unused slots are DBL_MAX. |
| dTargetPosition[2][MAX_NO_PLAYBACK_NAV] | double[2][15] | deg | [0][i]=target latitude, [1][i]=target longitude. |
| dGpsVruComp[2][MAX_NO_GPS] | double[2][4] | m | GPS VRU compensation offsets [x,y]. |
| dGpsXYDist[2][MAX_NO_GPS] | double[2][4] | m | GPS movement distance from roll/pitch [x,y]. |
| dGyroHeading[MAX_NO_GYRO] | float[5] | deg | Gyro headings. Unused slots are FLT_MAX. |
| lGyroStatus[MAX_NO_GYRO] | short[5] | SensorStatus | Gyro status. See SensorStatus. Unused slots are -1. |
| bGyroNotAccepted[MAX_NO_GYRO] | bool[5] | boolean | Gyro not-accepted flags. |
| dVRU[3][MAX_NO_VRU] | float[3][5] | deg, deg, m | [0]=roll, [1]=pitch, [2]=heave. Unused slots are FLT_MAX. |
| lVRUStatus[MAX_NO_VRU] | short[5] | SensorStatus | VRU status. See SensorStatus. Unused slots are -1. |
| dWind[2][MAX_NO_WIND] | float[2][5] | m/s, deg | [0]=wind speed, [1]=wind direction. Unused slots are FLT_MAX. |
| lWindStatus[MAX_NO_WIND] | short[5] | SensorStatus | Wind sensor status. See SensorStatus. Unused slots are -1. |
| lExtForceSensor[2][MAX_NO_EXT_FORCE] | float[2][2] | t, deg | [0]=external force, [1]=external force direction. Unused slots are FLT_MAX. |
| lRotSensor[MAX_NO_ROT] | float[3] | deg/min | ROT sensor values. Unused slots are FLT_MAX. |
| dWindDirection | float | deg | Vessel mean wind direction. |
| dWindSpeed | float | m/s | Vessel mean wind speed. |
| dCurrentDirection | float | deg | Estimated current/external-force direction except wind. |
| dCurrentSpeed | float | m/s | Estimated current/external-force speed except wind. |
| dCarrotPosition[2] | double[2] | deg | [0]=latitude, [1]=longitude. |
| dCarrotHeading | float | deg | Carrot heading. |
| dPositionSetpoint[2] | double[2] | deg | [0]=latitude, [1]=longitude of carrot end position. |
| dHeadingSetpoint | float | deg | Heading setpoint / carrot end heading. |
| lThrusterType[MAX_NO_THRUSTER] | short[10] | ThrusterType | Thruster type. See ThrusterType. Unused slots are -1. |
| lThrusterStatus[MAX_NO_THRUSTER] | short[10] | ThrStatus | Thruster status. See ThrStatus. Unused slots are -1. |
| dRPMCommand[MAX_NO_THRUSTER] | float[10] | -1..1 | RPM command. Unused slots are FLT_MAX. |
| dRPMFeedback[MAX_NO_THRUSTER] | float[10] | -1..1 | RPM feedback. Unused slots are FLT_MAX. |
| dPitchCommand[MAX_NO_THRUSTER] | float[10] | -1..1 | Pitch command. Unused slots are FLT_MAX. |
| dPitchFeedback[MAX_NO_THRUSTER] | float[10] | -1..1 | Pitch feedback. Unused slots are FLT_MAX. |
| dRelativeThrustCommand[MAX_NO_THRUSTER] | float[10] | -100..100 % | Relative thrust command. Unused slots are FLT_MAX. |
| dRelativeThrustFeedback[MAX_NO_THRUSTER] | float[10] | -100..100 % | Relative thrust feedback. Unused slots are FLT_MAX. |
| dAzimuthCommand[MAX_NO_THRUSTER] | float[10] | deg | Azimuth command. Unused slots are FLT_MAX. |
| dAzimuthFeedback[MAX_NO_THRUSTER] | float[10] | deg | Azimuth feedback. Unused slots are FLT_MAX. |
| dThrPowerFeedback[MAX_NO_THRUSTER] | float[10] | kW | Thruster power feedback. |
| bAxisControl[3] | bool[3] | boolean | [0]=along, [1]=athwart, [2]=heading control enabled. |
| dForceCmd[3] | float[3] | t | Commanded vessel force [surge,sway,yaw]. |
| ForceFdb[3] | float[3] | t | Feedback force [surge,sway,yaw]. |
| dDebugData[10] | float[10] | raw | Debug values. |
| sTCVoteStatus[6] | short[6] | bitmask | Vote result from TC1-TC6. See Vote Result Bitmask. |
| sDPCC_TCSCCInUse[3] | short[3] | device id | DPCC in-use values reported by TCSCC1-TCSCC3. See TCSCC/DPCC Status Values. |
| sTCSCC_DPCCInUse[3] | short[3] | device id | TCSCC in-use values reported by CC1-CC3. See TCSCC/DPCC Status Values. |
| sCCVoteBuffer[3][5] | short[3][5] | raw vote values | DPCC vote buffers from CC1-CC3. See Vote Buffer Values. |
| dCarrotVelocity[3] | float[3] | m/s, m/s, deg/min | Carrot velocity [surge,sway,yaw]. |
| dBallisticSpeed[3] | float[3] | m/s, m/s, deg/min | Ballistic speed [surge,sway,yaw]. |
| dControllerP[3] | float[3] | t | Controller proportional force/moment. |
| dControllerD[3] | float[3] | t | Controller derivative force/moment. |
| dControllerI[3] | float[3] | t | Controller integral force/moment. |
| dControllerWindFF[3] | float[3] | t | Wind feed-forward force/moment. |
| dControllerAccFF[3] | float[3] | t | Acceleration feed-forward force/moment. |
| dGeneratorLoad[MAX_NO_GENERATOR] | float[10] | kW | Generator load. |
| dDieselLoad[MAX_NO_MAIN_ENGINE] | float[4] | kW | Main engine/diesel load. |
| bGenBreakerStatus[MAX_NO_GENERATOR] | bool[10] | boolean | Generator breaker status. |
| bThrBreakerStatus[MAX_NO_THRUSTER] | bool[10] | boolean | Thruster breaker status. |
| bBusBreakerStatus[MAX_NO_BUSES] | bool[8] | boolean | Bus breaker status. |
| lNoOfGenConnected[MAX_NO_BUSES] | short[8] | count | Number of generators connected per bus. |
| lNoOfThrConnected[MAX_NO_BUSES] | short[8] | count | Number of thrusters connected per bus. |
| sCCPrefCC[3] | short[3] | bitmask | Preferred CC values from CC1-CC3. See Preferred CC Bitmask. |
| lCCInUse | short | device id | CC in use. Usually CC1=26, CC2=27, or CC3=28; 0 if not set. |
| lTCSCCInUse | short | device id | TCSCC in use. Usually TCSCC1=100, TCSCC2=101, or TCSCC3=102; 0 if not set. |
| lOSInCmd | short | device id | OS in command. Usually OS1=1 through OS25=25; 0 if not set. |
| lEstimatorThrForce[3] | float[3] | t | Estimator thruster force. |
| lEstimatorBiasForce[3] | float[3] | t | Estimator bias force. |
| lEstimatorDragForce[3] | float[3] | t | Estimator drag force. |
| lEstimatorDampForce[3] | float[3] | t | Estimator damping force. |
| lEstimatorWindForce[3] | float[3] | t | Estimator wind force. |
| lEstimatorExtForce[3] | float[3] | t | Estimator external force. |
| lJoystickCmd[3] | float[3] | -100..100 % | Joystick command on all axes. |
| lOpPointNumber | short | id | Operating point number. See Config-Dependent IDs. |
| lOPPointPos[2] | float[2] | m | Operating point position. |
| dPIDGainSettings[3] | float[3] | % | Position, velocity, integral gain set in MTOS. |
| dAlarmLimit | float | m | Position alarm limit. |
| lFTMode | short | enum/id | Follow-target mode. See Follow Target Mode. |
| lFTTarget1 | short | id | Follow-target target 1. See Config-Dependent IDs. |
| lFTTarget2 | short | id | Follow-target target 2. See Config-Dependent IDs. |
| lFTAdmin1 | short | id | Follow-target administrator 1. See Config-Dependent IDs. |
| lFTAdmin2 | short | id | Follow-target administrator 2. See Config-Dependent IDs. |
| lFTTransponderID | short | id | Follow-target transponder id. See Config-Dependent IDs. |
| lFTHPRAdminID | short | id | Follow-target HPR administrator id. See Config-Dependent IDs. |
| bRelHdgEnabled | bool | boolean | Relative heading enabled. |
| bFTCircleEnabled | bool | boolean | Follow-target circle enabled. |
| dFTCircleRadius | float | m | Follow-target circle radius. |
| dFTCirclePos[2] | float[2] | m | Follow-target circle position. |
| bAllocCombinatorMode | bool | boolean | Allocation combinator mode. |
| bEngineInVariableMode[MAX_NO_MAIN_ENGINE] | bool[4] | boolean | Main engine variable mode flags. |
| bBatteryReady[MAX_NO_GENERATOR] | bool[10] | boolean | Battery/generator ready flags. |
| dStateOfCharge[MAX_NO_GENERATOR] | float[10] | 0..1 | Battery state of charge. |
| dRemainingEnergy[MAX_NO_GENERATOR] | float[10] | kWh | Remaining energy. |
| dRemainingTimeMin[MAX_NO_GENERATOR] | float[10] | min | Remaining time. |
| dRemainingTimeMinWorstCase[MAX_NO_GENERATOR] | float[10] | min | Worst-case remaining time. |
| dGeneratorFuel[MAX_NO_GENERATOR] | float[10] | liter | Generator fuel. |
| lSpeedSensor[2][MAX_NO_SPEED] | float[2][3] | m/s | [0]=along, [1]=athwart. Unused slots are FLT_MAX. |
| dVoithLongitudalPitchCommand[MAX_NO_THRUSTER] | float[10] | -1..1 | Voith longitudinal pitch command. |
| dVoithTransversePitchCommand[MAX_NO_THRUSTER] | float[10] | -1..1 | Voith transverse pitch command. |
| dVoithLongitudalPitchFeedback[MAX_NO_THRUSTER] | float[10] | -1..1 | Voith longitudinal pitch feedback. |
| dVoithTransversePitchFeedback[MAX_NO_THRUSTER] | float[10] | -1..1 | Voith transverse pitch feedback. |
| dExtCtrlSpdRef[3] | float[3] | m/s | External control speed reference. |
| dExtCtrlAccRef[3] | float[3] | m/s^2 | External control acceleration reference. |
| bAutonomyMode | bool | boolean | Autonomy mode active. |
| trackingControllerData | track_controller_struct | nested struct | Track controller data; see below. |
| dRollStabilisationCommand[MAX_NO_THRUSTER] | float[10] | command | Roll stabilisation command per thruster. |
| i64TimeStamp | SYSTEMTIME | Windows SYSTEMTIME | Timestamp. |
| dwCRC | short | checksum/raw | CRC field. |
| dwBytePattern1 | long | raw | Byte-pattern marker. |
| dwBytePattern2 | long | raw | Byte-pattern marker. |

## track_controller_struct

Nested field in remas_struct as trackingControllerData.

| Field | C type | Format / unit | Notes |
| --- | --- | --- | --- |
| dHeadingPMoment | float | moment/force | Heading proportional moment. |
| dHeadingIMoment | float | moment/force | Heading integral moment. |
| dHeadingDMoment | float | moment/force | Heading derivative moment. |
| dXTDPMoment | float | moment/force | Cross-track proportional moment. |
| dXTDIMoment | float | moment/force | Cross-track integral moment. |
| dXTDDMoment | float | moment/force | Cross-track derivative moment. |
| dYawRateFF | float | feed-forward | Yaw-rate feed-forward. |
| dTotalMoment | float | moment/force | Total moment. |
| dCrabAngle | float | deg | Crab angle. |
| bInTurn | bool | boolean | In-turn flag. |
| dCrabInTurn | float | deg | Crab angle in turn. |
| dTurnRadius | float | m | Turn radius. |
| dXTE_used | float | m | Cross-track error used by controller. |
| dXTE_raw | float | m | Raw cross-track error. |
| dXTE_reference | float | m | Cross-track reference. |
| dHeadingReference | float | deg | Heading reference. |
| dYawRateReference | float | deg/min | Yaw-rate reference. |
| dWheelOverDistance | float | m | Wheel-over distance. |

## Legacy/Related Structs

### remas_struct2

remas_struct2 is another packed Remas layout defined in MTStructDef.h. It uses long and double for many fields and contains a smaller field set. The current RemasLogger UDP send path does not instantiate or send this struct.

### playback_struct

playback_struct is a packed binary playback/logging layout. It starts with SYSTEMTIME i64TimeStamp, then largely follows the same field order and units as remas_struct, with additional playback-specific fields such as:

lRotStatus[MAX_NO_ROT]

dPosCarrotVelocitySetpointFrame[2]

system filter values (dReferenceSystemFilter, dHeadingSystemFilter, dWindSystemFilter, dVRUSystemFilter)

simulated thruster feedback arrays

nested Seismic_Track_Data_struct, speed_controller_struct, and ilos_track_controller_struct

network node status arrays

This struct is used by playback/binary logging code, not by the active RemasLogger UDP output.