# MTIBS Tag Pattern Guide

This guide describes the patterns found in MTIBS telegram definitions for IBS shore data. It is based on the eight vessel GenSheet folders listed in README.md.

## What MTIBS Sends

MTIBS is an MT proprietary telegram used to send IBS data to shore. The List tab in Gmr100.xls or Gmr100PrilogSetup.xls defines how many MTIBS telegrams are sent and which tags are included in each telegram.

A typical row looks like:

$MTIBS1,1,1,1,1,1,1,139.58,88.80,1,1,1,1,1,1,1,1,1,1,1,1,0,0,1,1,1,1,1,1,1,1,1,1,1*4E

The first column is usually the telegram name, the second is the telegram number, and the remaining cells are comma-separated tag definitions.

## List Token Format

| Token part | Meaning |
| --- | --- |
| T | Data point/tag entry. |
| B | Boolean value. Interpret as 0/1, commonly closed/open, off/on, normal/alarm, or false/true depending on the tag description. |
| R | Real value. Interpret as a numeric analog/derived value. Units must be resolved from the Hxxx workbook or project context. |
| <tag> | The tag name to cross-reference in Hxxx sheets. |

Examples:

| Token | Interpretation |
| --- | --- |
| T,B=FUEL | Boolean tag named FUEL. The description sheet must define whether 1 means open, running, alarm, etc. |
| T,R=601_10001_LOAD | Real value for the 601_10001 equipment/object with suffix LOAD. |
| T,B=326_00051_ZS1 | Boolean status/switch point for equipment/object 326_00051. |

## Tag Name Families

The List tabs contain three broad tag families.

| Family | Shape | Examples | Notes |
| --- | --- | --- | --- |
| Structured numeric tags | <system>_<equipment>_<suffix> or <system>_<equipment> | 326_00051_ZS1, 351_10001_LT1, 601_10001_FORATE | Most common for IO, tanks, motors, engines, pumps, valves, alarms, and analog values. |
| Named project tags | Words plus numbers | Ballast1, DryBulk10, DENS_FO_N1 | Usually resolved in object/helper sheets such as BVAL, DATREAL, DATBOOL, or project-specific sheets. |
| Object/property tags | Dot-separated object fields | EMSDG1.Pow, EMSDG1.Status.CalcRunning, EMS_Main.Speed | Usually derived EMS/energy-monitoring values rather than direct IO points. |

## Structured Numeric Tags

Structured tags usually split like this:

326_00051_ZS1
|   |     |
|   |     suffix / signal role
|   equipment or object number
system or process family code

### System / Process Code

The first three digits group tags by vessel system or application area. The code is not enough by itself; always verify in the Hxxx workbook because meanings can vary by vessel and project.

Observed examples:

| Code | Observed descriptions / context |
| --- | --- |
| 326 | Dry bulk compressors, dry bulk tanks, dry bulk cooling pumps. |
| 351 | Fuel oil tanks, fuel oil transfer, fuel oil day tanks. |
| 352 | Dirty oil / oil transfer examples. |
| 357, 358, 359 | Tank/level and utility values in several vessels. |
| 404, 626 | Engine / machinery values in 280/312 class examples. |
| 601 | Main engine/thruster/engine fuel and status values in several examples. |
| 610 | Generators. |
| 803 | Bilge/cargo/bilge alarm examples. |
| 867, 871, 873 | Electrical/UPS/power-related alarms and status examples. |
| 900 | Common/system alarms or generic system points. |

Treat this as a lookup hint, not a contract. The authoritative text is the matching Descr in the Hxxx workbook.

### Equipment / Object Number

The middle segment is usually a five-character equipment/object number, for example 10001, 20001, 00051, or 30002.

Common patterns:

| Pattern | Typical meaning |
| --- | --- |
| 0xxxx | Common, centerline, shared, or non-side-specific object. |
| 1xxxx | Port / PS / No. 1 side or first equipment group. |
| 2xxxx | Starboard / SB / No. 2 side or second equipment group. |
| 3xxxx and higher | Additional tanks, trains, switchboards, cargo/dry-bulk groups, or project-specific groupings. |
| last two or three digits | Equipment sequence within the system/group. |

Examples from Hxxx sheets:

| Base tag | Description evidence |
| --- | --- |
| 326_00051 | Dry bulk air compressor 1 / DB air comp. 1. |
| 351_10001 | Ship fuel / fuel oil 1-P tank level. |
| 601_10001 | Main engine / PME / thruster-related values depending on vessel. |

### Suffix / Signal Role

The suffix describes the specific point for a base object. It often follows ISA-style instrumentation letters, but project-specific suffixes are common.

| Suffix | Observed meaning / use | Typical type in MTIBS |
| --- | --- | --- |
| _PT | Pressure transmitter / pressure analog value. | T,R |
| _TT | Temperature transmitter / temperature analog value. | T,R |
| _LT, _LT1, _LT2 | Level transmitter / tank level. TK rows often reference LT1/LT2 columns. | T,R |
| _JT | Power/load/current-derived machinery value in engine/electrical contexts. | T,R |
| _IT, _ST, _SI | Current/speed/status analog values depending on sheet and description. | T,R |
| _FORATE | Fuel or flow consumption rate. | T,R |
| _LOAD, _PCT_LOAD | Load or percent load. | T,R |
| _HR, _HOUR, _RH | Hourmeter or running-hours value. | T,R |
| _XA | Alarm/fail indication. Examples include overload, VFD fault, high tank alarm. | T,B |
| _XAX | Alarm variant / extended alarm point. Verify description. | T,B |
| _XC, _XC_, _XCB, _XC2, _XC3 | Command/control/status variant. Verify source column and description. | Usually T,B |
| _XI, _XI_, _XI1 | Indication/status value. Type can vary by source. | Usually T,B or T,R depending on List token. |
| _XY1, _XY2 | Output command points, often start/stop/open/close/reset commands. | T,B |
| _ZS1 | Status/switch point; often running, closed, or position status. | T,B |
| _ZSH, _ZSL, _ZZH | High/low/limit switch status. | T,B |
| _HS1 | Hand/local switch or local status. | T,B |
| _COMFAIL, _NO_CONN | Communication failure / no connection indicator. | T,B or T,R as listed |
| _SD1, _SD2, _SD3 | Shutdown/status bits, project-specific. | T,B |

The List token type (B or R) is authoritative for the value format sent in MTIBS, even when a suffix is reused in different contexts.

## How To Resolve a Tag

Use this workflow for dashboard mapping or decoding.

Open Gmr100.xls or Gmr100PrilogSetup.xls and find the List tab.

Collect all T,B=<tag> and T,R=<tag> entries from MTIBS rows.

Search the Hxxx detail workbooks for exact TagName = <tag>.

If exact search fails, strip the final suffix and search the base tag, for example search 326_00051 for 326_00051_ZS1.

Use the row's TaskName, Descr, and source/reference columns to interpret the point.

Use the List token type for the transmitted value format: B boolean or R real.

## Useful Hxxx Sheets

| Sheet family | What it usually contains | Useful columns |
| --- | --- | --- |
| AIS, AIS02 | Analog input or analog source values. | TaskName, TagName, Descr, PhysRange, alarm limits. |
| DIS, DIS02 | Digital input/status/alarm points. | TagName, Descr, NormPos, communication address/bit. |
| DOS | Digital output/command points. | TagName, Descr, command source. |
| AOS | Analog outputs. | TagName, Descr, range/source columns. |
| BVAL | Valve objects. | Base TagName, Descr, Open, Close, Opened, Closed, Local. |
| TK | Tank objects. | Base TagName, Descr, LT1, LT2, density and alarm columns. |
| MOT | Motor/pump/compressor objects. | Base TagName, Descr, Start, Stop, Running, Fail, Local. |
| RunningHours | Derived running-hour values. | TagName, Descr, Input1, Value. |
| DATREAL, DATBOOL, DATINT | Constants or derived/shared data values. | TagName, Descr, Value. |
| EMSMain, EMSDG, EMSIntegrator | Energy monitoring derived objects. | Object TagName, Descr, input/output property columns. |
| MBTCP*, MBRead*, BoolProvider32 | Communication mapping/source providers. | Modbus/register/bit mapping columns. |

## Evidence Examples

These examples were cross-referenced from the List tags into Hxxx detail sheets.

| List tag | Hxxx evidence | Meaning inferred |
| --- | --- | --- |
| 326_00051_HS1 | DIS02 row: Descr=DB PORT AIR COMP. 1 LOCAL STAT; MOT base row references Local=gv326_00051_HS1.IO. | Boolean local/hand status for dry bulk air compressor 1. |
| 326_00051_ZS1 | DIS02 row: Descr=DB PORT AIR COMP. 1 RUN STAT; MOT base row references Running=gv326_00051_ZS1.IO. | Boolean running/status switch for the motor object. |
| 326_00051_XA | DIS row: overload/fail alarm; MOT base row references Fail=gv326_00051_XA.IO. | Boolean alarm/fail point. |
| 326_00051_XY1 | DOS row: start output; MOT base row references Start=gv326_00051_XY1. | Boolean start/output command. |
| 326_00051_HOUR / 326_00051_HR | DATREAL / RunningHours rows describe compressor hours and use ZS1 running status as input. | Real running-hours value. |
| 351_10001_LT1 | AIS02 row describes ship fuel 1-P tank level; TK base row 351_10001 references LT1=gv351_10001_LT1. | Real tank level transmitter value. |
| 601_10001_PT | AIS rows describe fuel oil pressure or emergency air pressure depending on vessel. | Real pressure value; description is vessel-specific. |
| 601_10001_FORATE | AIS02 row: fuel consumption rate. | Real flow/fuel consumption rate. |
| 601_10001_XA | DIS row: VFD fault alarm in one vessel. | Boolean alarm/fault. |

## Important Caveats

The tag structure is consistent enough for discovery, but not enough for final meaning without the Hxxx workbook row.

The same system code can describe different equipment on different vessel classes.

TaskName values such as Normal, Slow, HighCom, Pumps, BVAL, and Modbus are useful hints about update rate/source, not a full semantic definition.

Some tags in the List are derived application values, not direct IO points. Examples include EMSDG1.Pow, EMS_Main.Speed, DENS_FO_N1, and Ballast1.

The Hxxx source row may be a base object row rather than the exact List tag row. In that case use the object reference columns to map suffixes.